# Using HW 7 Work: AVL Proof

clear

clf

% CSV file info

csvFiles = {'NACA4414.csv','NACA1414.csv','re6.csv','re7.csv'};

scriptDir = fileparts(matlab.desktop.editor.getActiveFilename);

% Loop through each CSV file and import the data

for i = 1:numel(csvFiles)

% Full path of the current CSV file (assuming it's in the same directory)

csvFilePath = fullfile(scriptDir, csvFiles{i});

% Read the CSV file as a table

T = readtable(csvFilePath);

% Get the variable names from the first row

varNames = T.Properties.VariableNames;

% Loop through each column and create variables in the workspace

for j = 1:numel(varNames)

% Extract the data, excluding the first row

data = T.(varNames{j})(1:end);

% Assign each column to a variable with the corresponding name

assignin('base', varNames{j}, data);

end

end

Setting Constants

Vinf = 1;

Vinf = 1;

rho = 1;

% % % PART _ % % %

%  some code

% % % PART _ % % %

c = (max(NACA4414_X) - min(NACA4414_X));

ZeroLiftAlfa = lift(NACA4414_X,NACA4414_Z,-3.97,c,Vinf,rho,false,false)

function [Lift_Coef] = lift(x,z,alpha,c,Vinf,rho,plooot,PartA)

% Setting Variables

N = length(x) - 1;

xj = x;

zj = z;

ni = ones(N,2);

A = zeros(N,N);

F = zeros(N,1);

% Setting control points as the mid point between each given node. Number

% of Nodes - 1 = Number of Controll Points

for i=1:N

xi(i) = (xj(i) + xj(i+1)) ./ 2;

zi(i) = (zj(i) + zj(i+1)) ./ 2;

end

% Getting the Nomal Vectors for use later

for j=1:N

dx(j) = xj(j+1) - xj(j);

dz(j) = zj(j+1) - zj(j);

len = sqrt((dx(j)^2) + (dz(j)^2));

ni(j,:) = [-dz(j)/len , dx(j)/len];

end

% Setting the Influence Coefficents (A) and the vorticies

for i=1:N

for j=1:(N+1)

dx = xi(i) - xj(j);

dz = zi(i) - zj(j);

r2 = dx .* dx + dz .* dz;

A(i, j ) = 1 ./ (2 * pi * r2) .* (dz *  ni (i,1) - dx * ni(i ,2));

end

F(i) = -Vinf .* ( cosd(alpha) .* ni(i,1) + sind(alpha) .* ni(i ,2));

end

% Forcing the Kuda Conditon

A(N+1,1) = 1;

A(N+1,N+1) = 1;

F(N+1) = 0;

% % % PART A % % %

% solving the system

Gamma = A\F;

if (PartA)

Gamma

end

% % % PART A % % %

% % % PART B % % %

% Calculating the Lift and Lift Coeff

L = rho * Vinf * sum(Gamma)  * c;

Lift_Coef = L / (0.5 * rho * Vinf.^2 * c);

% % % PART B % % %

% % % PART D % % %

% Boolian check for whether or not to plot the streamlines for the Bonus

% Question

if (plooot)

figure(1); clf ;

[Y,X] = meshgrid(linspace(-0.5, 0.5, 51),linspace(-0.5, 1.5, 101));

Z = X + Y*1i;

F = Z*Vinf*(cosd(alpha) - 1i*sind(alpha));

for j = 1:N, F = F + Gamma(j)*1i/(2*pi) * log(Z - xj(j) - zj(j)*1i); end

Psi = imag(F);

contour(X,Y,Psi,25); hold on;

plot(xj, zj , 'k-', 'linewidth' , 2); axis equal;

u = ni(:,1);

v = ni(:,2);

xi = xi.';

zi = zi.';

%quiver(xi,zi,u,v % BugChecking for Normal Vectors

hold off

end

% % % PART D % % %

end