# Define Parameters

% Take off

% Drag

clear

clf

W_pounds.Wi = 255000;

W_pounds.Wp = 135896;

W_pounds.Wf = W_pounds.Wi - W_pounds.Wp;

Power_Horses = 4637 * 4;

W.i = 115666.054;

W.p = 61641.3887; % IN POUNDS

W.f = W.i - W.p;

Power = 3410000 *4; %K Watts

ThrustEff = .65;

% Aircraft Wing Param

c = 2; % Cord Length (Constant for all surfaces)

b = [10,10,4,4,4]; % legth of each half wing

Recrit = 10^6;

% Defining Fuselage Parameters

Fuselage.x1 = 0; % Starting x bound

Fuselage.x2 = 35.36; % Ending x bound

Fuselage.y1 = 0; % Starting y bound

Fuselage.y2 = 13.41; % Starting y bound

% Defining Conditions

%   Cruise

conditions.cruise.alt = 6705.6;

conditions.cruise.rho = 0.61003;

conditions.cruise.U_inf = 180;

conditions.cruise.v = 2.602e-5;

conditions.cruise.SoS = 313.5318;

conditions.cruise.Re = 6.9887e+6;

%   Sea Level

conditions.sea.alt = 0;

conditions.sea.rho = 1.2250; % Density at altitude

conditions.sea.U_inf= 65; % Velocity of travel

conditions.sea.v = 0.000014607; % Kinematic Viscosity

conditions.sea.SoS = 340.294;

conditions.sea.Re = 4.4499e+6;

%   Landing Conditions

conditions.landing.alt = 0;

conditions.landing.rho = 1.2250; % Density at altitude

conditions.landing.U_inf= 62.5265; % Velocity of travel

conditions.landing.v = 0.000014607; % Kinematic Viscosity

conditions.landing.SoS = 340.294;

conditions.landing.Re = 4.2805e+6;

% Wing and Elevator Object

%   Cruise

aerodynamics.cruise.C_l = .24;

aerodynamics.cruise.C_l_max = 1.0826;

aerodynamics.cruise.C_d = [0.00603,0.00603,0.00574,0.00574,0.00574];

%   Sea Level

aerodynamics.sea.C_l = 1.1; % Combine C_l

aerodynamics.sea.C_l_max  = 1.7389;

aerodynamics.sea.C_d = [0.00519,0.00519,0.00591,0.00591,0.00591]; % C_d per half wing

%   Landing Aerodynamics

aerodynamics.landing.C_l = 1.19; % Combine C_l

aerodynamics.landing.C_l_max  = 1.7389;

aerodynamics.landing.C_d = [0.01062,0.01062,0.00869,0.00869,0.00869]; % C_d per half wing

Check for speeds for lift = weight

U_inf_match = Find_Lift_is_Weight(Fuselage, aerodynamics.sea, conditions.sea, c, b, W.i)

Take-off Conditions Check

[Sea.Drag,Sea.Lift] = GetTotalAerodynamics(Fuselage,aerodynamics.sea,conditions.sea,c,b);

Sea.Lift_to_Weight = Sea.Lift / W.i;

Sea.Thrust = CalcThrust(conditions.sea,Power,ThrustEff);

Sea.Thrust_to_Drag = Sea.Thrust / Sea.Drag;

[Sea.IsStall,Sea.VStall] = CheckStall(conditions.sea,aerodynamics.sea,(b(1)*2*c),W.i);

Sea.U_inf = conditions.sea.U_inf;

Sea.Mach = Sea.U_inf / conditions.sea.SoS;

Sea.alfa = 3.0;

Sea.X_np = 4.68;

conditions.sea

Sea

Cruise Conditions Check

[Cruise.Drag,Cruise.Lift] = GetTotalAerodynamics(Fuselage,aerodynamics.cruise,conditions.cruise,c,b);

Cruise.Lift_to_Weight = Cruise.Lift / W.i;

Cruise.Thrust = CalcThrust(conditions.cruise,Power,ThrustEff);

Cruise.Thrust_to_Drag = Cruise.Thrust / Cruise.Drag;

[Cruise.IsStall,Cruise.VStall] = CheckStall(conditions.cruise,aerodynamics.cruise,(b(1)*2*c),W.i);

Cruise.U_inf = conditions.cruise.U_inf;

Cruise.Mach = Cruise.U_inf / conditions.cruise.SoS;

Cruise.alfa = -0.6297;

Cruise.X_np = 4.54;

conditions.cruise

Cruise

Landing Conditions Check

% A = 10

% Flaps = 15

% Speed = 1.2 Stall

[Landing.IsStall,Landing.VStall] = CheckStall(conditions.landing,aerodynamics.landing,(b(1)*2*c),W.i);

conditions.landing.U_inf = (1.2 * Landing.VStall); % Setting Landing conditions to 1.2 times stall

Landing.U_inf = conditions.landing.U_inf;

[Landing.Drag,Landing.Lift] = GetTotalAerodynamics(Fuselage,aerodynamics.landing,conditions.landing,c,b);

Landing.Lift_to_Weight = Landing.Lift / W.i;

Landing.Thrust = CalcThrust(conditions.landing,Power,ThrustEff);

Landing.Thrust_to_Drag = Landing.Thrust / Landing.Drag;

[Landing.IsStall,Landing.VStall] = CheckStall(conditions.landing,aerodynamics.landing,(b(1)*2*c),W.i);

Landing.Mach = Landing.U_inf / conditions.landing.SoS;

Landing.alfa = 8;

Landing.X_np =4.74;

conditions.landing

Landing

Range Check

Range_At_Cruise_in_miles = RangeCalc(W_pounds,Power_Horses,conditions.cruise)